California State University, Fresno 
Lyles College of Engineering 
Electrical and Computer Engineering Department 
 
TECHNICAL REPORT 
 
Assignment:  Number 5
Experiment Title: HW/SW Co-Design of an Embedded System on FPGA
Course Title: ECE 178 (Embedded Systems)  
Instructor: Dr. Reza Raeisi  
  
 Prepared by: Anthony Herrick 
 Student ID #300144976
Date Submitted: 10/09/2022

INSTRUCTOR SECTION 
 
Comments:  _______________________________________________________________ 
  _______________________________________________________________ 
  _______________________________________________________________ 
  _______________________________________________________________ 
  _______________________________________________________________ 
  _______________________________________________________________ 
  _______________________________________________________________ 
 
Final Grade:  ____________________

TABLE OF CONTENTS

Objective	4

Hardware Requirements	4

Software Requirements	4

Background	4

Project Overview	7

Project Procedure	8

Conclusion	30

Appendix A (Part 1 Code)	31

Appendix B (Part 2 Code)	33

Appendix C (Part 3 Code)	36

Appendix D (Part 4 Code)	37

Appendix E (Part 5 Code)	40

LIST OF FIGURES

1.	Soft Core Embedded System Design	5

2.	7-Segment Display Address Spaces	6

3.	7-Segment Display Breakdown	7

4.	Register Values Before Call	8

5.	After Entering the Divide Subroutine	9

6.	Values After the First CONT loop	9

7.	Part 1 Final Values	10

8.	Value of Switches being loaded into r4	11

9.	Loading the Byte Sized Value for the 7 Segment	11

10.	Board After Value is Loaded into 7-Segment	12

11.	Squared Value on the LEDs	13

12.	After SRLI	14

13.	After And	15

14.	After r10 is iterated	16

15.	Starting Values Entering the Fourth Loop	17

16.	Fourth Loop After srli and and	18

17.	End of Part Three Program Registers	19

18.	Part 3 Test Case Results	20

19.	Registers before entering Ones Loop	21

20.	Registers after exiting Ones Subroutine	22

21.	Before Entering Ones subroutine Second Time	23

22.	After Exiting Ones Subroutine	24

23.	Final Values Of Program	25

24.	7 Segment Part 4	26

25.	Starting values after Initialization	27

26.	Registers after Returning from Count	28

27.	Part 5 LEDs	29

# Objective

Upon the completion of this lab, one will have co-designed an embedded soft-core system and application development in an FPGA design environment. We will be using Intel’s EDA design tool Qsys, in conjunction with the Quartus software, and Intel FPGA monitor.

# Hardware Requirements

- Computer with Intel FPGA Monitor program 16.1

- Computer with Quartus Prime 16.1.

- DE2-115 Board

- A-B USB Cable

# Software Requirements

- Intel FPGA monitor program 16.1 or greater

- Quartus Prime with Qsys, version 16.1.

# Background

For this lab, we will be using the soft-core embedded system that was designed in the previous lab. The block diagram for this embedded system can be seen in the figure below.

## Soft Core Embedded System Design

In order to write to a 7-segment display, we have two address spaces, one is for the four most significant bits and the second for the four least significant bits. This is because the DE2-115 board has eight 7-Segment displays. From the design in the last lab, the address spaces for these were, 0x2020 and 0x2030. The typical address spaces for the DE2-115 board can be seen below.

## 7-Segment Display Address Spaces

Now, from this, we also need to know how to manipulate the 7-Segments in the way that we want. So, being that the display is an active low device, in order to light up a certain display we must pass a 1 into the bit that we want to light up. Now, the segments are controlled individually, so, passing a 7 bit value of 0b1111000 will turn on the bits corresponding to bit zero, one, and two. How these bits correspond to the actual 7 segment display can be seen below.

## 7-Segment Display Breakdown

With this knowledge we can begin this project.

# Project Overview

For this lab, we will be using the soft-core embedded system that was developed in the previous project in order to perform a variety of tasks on the DE2-115 board. The first of these tasks are converting a binary number to its decimal equivalent. Then, we will be creating a loop that will read the DIP switches, output the value on the 7-segment display, create a delay, and display the square on the LEDs. Next, we will be using a program to find the longest string of ones in a given word value. Then using this to work for a list of word values. And lastly creating a counter that works in the bits 10-15 on a given value.

.

# Project Procedure

# Conclusion

This lab accomplished its goal of using the softcore embedded system formed in the last exercise, and Intel’s FPGA design enviroment in order to complete certain tasks. We performed binary to decimal conversions, displayed an input value from the switches on the 7-Segment Display, and the square of that value on the LEDs, counted the longest string of ones from a list, and created a counter in certain memory bits. This further bolstered the knowledge of PIO devices and how they work in a  NIOS II coding enviroment.

# Appendix A (Part 1 Code)

/* Program that converts a binary number to decimal, you can modify the program */

.text

.global    _start

_start:

movia r4, N

addi   r8, r4, 4    # r8 points to storage location

ldw   r4, (r4)     #r4 loads N

call   DIVIDE    #parameter for DIVIDE is in r4

stb   r3, 1(r8)

stb   r2, (r8)

END:  br  END

#Subroutine can be modified if needed*/

# Subroutine to perform the integer division r4 / 10 */

# Returns: quotient in r3, and remainder in r2 */

DIVIDE:

mov  r2, r4     # will be the remainder

movi  r5, 10    #divisor

movi  r3, 0    #r3 will be the quotient

CONT:

blt  r2, r5, DIV_END

sub  r2, r2, r5

addi  r3, r3, 1

br  CONT

DIV_END:

ret

N:   .word  9876      #the decimal number to be converted

Digits:  .space 4       #space for storage location

.end

# Appendix B (Part 2 Code)

.data

LUT:

.byte 0b11000000#0

.byte 0b11111001#1

.byte 0b10100100#2

.byte 0b10110000#3

.byte 0b10011001#4

.byte 0b10010010#5

.byte 0b10000010#6

.byte 0b11111000#7

.byte 0b10000000#8

.byte 0b10010000#9

.text

.equ	RLEDs,  0x2040

.equ	SWITCHES, 0x2010

.equ	GLEDs, 0x2030

.equ	HexLSB, 0x2020

.equ	HexMSB, 0x2000

.global _start

_start:

movia	r2, RLEDs		#LED Add

movia	r3, SWITCHES	#Switch Add

#movia	r5, GLEDs		#Green LEDs

movia	r6, HexLSB		#LSB Hex Bits

movia	r5, 0x500		#LUT

movia 	r13, 0x1

br		LOOP

DELAY:

movia r11, 0x0        #Initialize R11, our iterator

movia r10, 0x04C4B40     #what we have to iterate to

Delay2:

bgt r11, r10, LOOP2 #Compares iteration 1 instructino

add r11 ,r11, r13 #1 #Adds one to the iteration, 2 instructions

movia r12, 0x0 # 3 instructions (Filler code)

add r12, r12, r12 # 4 instructions (FIller Code)

br Delay2

LOOP:

ldbio	r4, (r3)		#Switch State into R4

add 	r4, r4, r5

ldbio	r7, (r4)

stbio	r7, (r6)

br DELAY

LOOP2:

ldbio 	r4, (r3)

mul		r4, r4, r4

stwio	r4, (r2)

br	LOOP

.end

# Appendix C (Part 3 Code)

.text

.global _start

_start:

ldw  r9, TEST_NUM(r0)

mov  r10, r0

LOOP: 	beq  r9, r0, END

srli  r11, r9, 0x01

and  r9, r9, r11

addi  r10, r10, 0x01

br  LOOP

END:

br  END

TEST_NUM:

.word 0x3fabedef

.end

# Appendix D (Part 4 Code)

.text

.global _start

_start:

movia  r9, TEST_NUM

mov  r10, r0

movia	 r6, 0

movia	 r2, 0

main:

ldw	r4, (r9)

bgt r6, r2, skip

mov r6, r2

skip:

movia	r2, 0

beq	r4, r0, end

call ONES

addi r9, r9, 4

br main

ONES:

beq  r4, r0, ENDONES

srli  r11, r4, 0x01

and  r4, r4, r11

addi  r2, r2, 0x01

br  ONES

ENDONES:

ret

end:

movia r1, 0x10000020

movia r7, LUT

add r7, r7, r6

ldb r6, (r7)

stb	r6, (r1)

ENDLOOP:

br ENDLOOP

TEST_NUM:

.word 0x3fabedef

.word 0xFF12384F

.word 0x99FCAB94

.word 0x10219341

.word 0xFCABEFCA

.word 0x12345678

.word 0xFFF00000

.word 0xF1231112

.word 0x00FEABCA

.word 0xF1234F87

.word 0x00000000

LUT:

.byte 0b11000000#0

.byte 0b11111001#1

.byte 0b10100100#2

.byte 0b10110000#3

.byte 0b10011001#4

.byte 0b10010010#5

.byte 0b10000010#6

.byte 0b11111000#7

.byte 0b10000000#8

.byte 0b10010000#9

.byte 0b10001000#A

.byte 0b10000011#b

.byte 0b11000110#C

.byte 0b10100001#d

.byte 0b10000110#E

.byte 0b10001110#F

.end

# Appendix E (Part 5 Code)

.text

.equ	RLEDs,  0x2040

.global _start

_start:

movia r3, 0b1111110000000000

movia r6, 0b11111111111111110000001111111111

movia r4, RLEDs

movia r1, 0x3fabedef

movia r2, 0b0001111110000000000

Loop:

and r2, r2, r1

call count

and r1, r1, r6

or r1, r1, r2

br Loop

count:

beq  r2, r3, reset

addi r2, r2, 0x400

stw r2, (r4)

ret

reset:

movia r2, 0x0

stw r2, (r4)

ret

END:

br  END

.end